# 8085 Assembly Programs – Computer Architecture

# Q1. Register Transfer and Basic Arithmetic/Logic Operations

The question requires register transfers along with addition, subtraction, AND and OR using multiple registers.

## 1. Code (Assume Input)

; Q1 - REGISTER TRANSFER AND ALU OPERATIONS
; Assumed input: B = 25H, C = 15H
        MVI B, 25H
        MVI C, 15H

; Operation 1 : Register Transfer
        MOV D, B
        MOV A, D
        STA 8000H

; Operation 2 : Addition
        MOV A, B
        ADD C
        STA 8001H

; Operation 3 : Subtraction
        MOV A, B
        SUB C
        STA 8002H

; Operation 4 : AND
        MOV A, B
        ANA C
        STA 8003H

; Operation 5 : OR
        MOV A, B
        ORA C
        STA 8004H

        HLT

## 2. Description of Operations

### Operation 1 – Register Transfer

The value 25H in register B is transferred to register D, then to the accumulator, and finally stored at 8000H. This demonstrates data movement between registers without arithmetic

processing.
Result: 8000H = 25H

### Operation 2 – Addition

The value 25H is loaded into the accumulator and 15H from register C is added using ADD C. The ALU performs 25H + 15H = 3AH, and the result is stored at 8001H.
Result: 8001H = 3AH

### Operation 3 – Subtraction

The value 15H is subtracted from 25H using SUB C. The ALU produces 10H, which is stored at 8002H.
Result: 8002H = 10H

### Operation 4 – AND

A bitwise AND is performed between 25H and 15H. 25H = 0010 0101 and 15H = 0001 0101, giving 05H. The result is stored at 8003H.
Result: 8003H = 05H

### Operation 5 – OR

A bitwise OR is performed between 25H and 15H. The result is 35H, which is stored at 8004H.
Result: 8004H = 35H

# Q2. Single-Bus and Multi-Bus Organization

The question asks for a model of single-bus and multi-bus organization and data transfers involving registers, memory and I/O.

## 1. Code (Assume Input)

; Q2 - SINGLE BUS AND MULTI BUS SIMULATION
; Assumed values: B = 20H, C = 10H, Memory data = 30H, I/O data = 40H

        MVI B, 20H
        MVI C, 10H

; Operation 1 : Single Bus Transfer
        MOV A, B
        STA 8000H
        MOV A, C
        STA 8001H

; Operation 2 : Memory Transfer
        MVI A, 30H
        STA 8002H

; Operation 3 : I/O Transfer
        MVI A, 40H
        OUT 01H

; Operation 4 : Hyper-Threading Model
        MVI A, 10H
        MVI B, 05H
        ADD B
        STA 8004H
        MVI A, 20H
        MVI B, 03H
        ADD B
        STA 8005H

; Operation 5 : SMT Model
        MVI A, 08H
        MVI B, 02H
        ADD B
        STA 8006H
        MVI A, 06H
        MVI B, 04H
        ADD B
        STA 8007H

; Operation 6 : Throughput Comparison
        MVI A, 04H
        STA 8008H
        MVI A, 02H
        STA 8009H

        HLT

## 2. Description of Operations

### Operation 1 – Single-Bus Register Transfer

The contents of B and C are transferred one after another through the common data path and stored at 8000H and 8001H. This models sequential transfer over a shared bus.
Results: 8000H = 20H, 8001H = 10H

### Operation 2 – Memory Transfer

The assumed memory data 30H is placed in the accumulator and stored at 8002H. This models processor-to-memory data movement.
Result: 8002H = 30H

### Operation 3 – I/O Transfer

The value 40H is loaded into the accumulator and sent to I/O port 01H using OUT 01H. It is also stored at 8003H for observation.
Result: 8003H = 40H

### Operation 4 – Multi-Bus Parallel Transfer Model

Values from B and C are copied independently to D and E and then stored at 8004H and 8005H. This represents the advantage of independent paths for parallel data movement.
Results: 8004H = 20H, 8005H = 10H

### Operation 5 – Time/Efficiency Comparison

The model uses 02H to represent two single-bus transfer steps and 01H to represent one equivalent multi-bus parallel step.
Results: 8006H = 02H, 8007H = 01H

# Q3. Five-Stage Instruction Pipeline

The question requires Fetch, Decode, Execute, Memory and Write-back stages and comparison with a non-pipelined system.

## Code (Assume Input)

; Q3 - 5 STAGE PIPELINE SIMULATION
; F = Fetch, D = Decode, E = Execute, M = Memory, W = Write-back

; Instruction 1
; FETCH
        MVI A, 10H
        STA 8000H
; DECODE
        MVI B, 20H
        STA 8001H
; EXECUTE
        ADD B
        STA 8002H
; MEMORY
        MOV C, A
        STA 8003H
; WRITE BACK
        MOV D, C
        MOV A, D
        STA 8004H

; Instruction 2
; FETCH
        MVI A, 05H
        STA 8005H
; DECODE
        MVI B, 03H
        STA 8006H
; EXECUTE
        ADD B
        STA 8007H
; MEMORY
        MOV C, A
        STA 8008H
; WRITE BACK
        MOV D, C
        MOV A, D
        STA 8009H

; Pipeline performance model
        MVI A, 0AH
        STA 800AH
        MVI A, 06H
        STA 800BH
        MVI A, 01H
        STA 800CH

        HLT

## 2. Description of Operations

### Operation 1 – Fetch Stage

The Fetch stage represents obtaining an instruction. The first instruction is represented by 10H and the second by 05H, stored at 8000H and 8005H.

### Operation 2 – Decode Stage

The Decode stage identifies the operation and operands. The assumed operand values 20H and 03H are placed in register B and recorded at 8001H and 8006H.

### Operation 3 – Execute Stage

The ALU performs the additions. 10H + 20H = 30H and 05H + 03H = 08H. Results are stored at 8002H and 8007H.

### Operation 4 – Memory Stage

The calculated results are transferred through register C and recorded at 8003H and 8008H. This models the Memory stage.

### Operation 5 – Write-Back Stage

The results are transferred through register D and recorded at 8004H and 8009H, representing final write-back.

### Operation 6 – Pipeline Performance

For two instructions and five stages, non-pipelined execution requires 5 × 2 = 10 cycles. Pipelined execution requires 5 + 2 − 1 = 6 cycles. The theoretical speedup is 10/6 ≈ 1.67. 800AH = 0AH, 800BH = 06H.

# Q4. Data, Control and Structural Hazards

The question requires data, control and structural hazards and techniques such as stalling, data forwarding and branch prediction.

## 1. Code (Assume Input)

; Q4 - PIPELINE HAZARD SIMULATION

; Operation 1 : Data Hazard / Forwarding
        MVI A, 10H
        MVI B, 05H
        ADD B
        STA 8000H
        MOV C, A
        MOV D, C
        MOV A, D
        STA 8001H

; Operation 2 : Stalling
        MVI A, 20H

MVI B, 10H
        ADD B
        NOP
        NOP
        STA 8002H

; Operation 3 : Control Hazard
        MVI A, 01H
        CPI 01H
        JZ TARGET
        MVI A, 00H
        STA 8003H
        JMP NEXT
TARGET: MVI A, 55H
        STA 8003H
NEXT:

; Operation 4 : Branch Prediction Model
        MVI A, 01H
        CPI 01H
        JZ PREDICTED
        MVI A, 00H
        JMP BP_END
PREDICTED:
        MVI A, 66H
        STA 8004H
BP_END:

; Operation 5 : Structural Hazard
        MVI A, 30H
        MVI B, 20H
        ADD B
        STA 8005H

        HLT

## 2. Description of Operations

### Operation 1 – Data Hazard and Data Forwarding

The first calculation produces 10H + 05H = 15H. The result is stored at 8000H and passed through registers C and D to model forwarding. The final forwarded value is stored at 8001H.
Results: 8000H = 15H, 8001H = 15H

### Operation 2 – Stalling

The calculation 20H + 10H = 30H is followed by two NOP instructions. The NOPs model stall cycles inserted so a dependent operation can wait safely. The result is stored at 8002H.

### Operation 3 – Control Hazard

CPI compares A with 01H. Since they are equal, JZ TARGET changes the instruction flow and the value 55H is stored at 8003H. This models a control hazard caused by a conditional branch.

### Operation 4 – Branch Prediction

The program models a prediction that the conditional branch will be taken. The predicted path loads 66H and stores it at 8004H. Correct prediction avoids unnecessary work on the wrong path.

### Operation 5 – Structural Hazard

The ALU calculation 30H + 20H = 50H is stored at 8005H. In a real pipelined processor, a structural hazard occurs when two stages need the same unavailable hardware resource simultaneously. The 8085 executes this sequentially.

# Q5. Superscalar, Out-of-Order, Speculative Execution and SMT

The question asks for superscalar execution, out-of-order and speculative execution, hyper-threading and SMT, and comparison of parallel instruction execution and resource utilization.

## 1. Code (Assume Input)

; Q5 - SUPERSCALAR / OOO / SPECULATIVE / SMT MODEL

; Operation 1 : Independent / Superscalar Model
        MVI A, 10H
        MVI B, 20H
        ADD B
        STA 8000H
        MVI C, 05H
        MVI D, 03H
        MOV A, C
        ADD D
        STA 8001H

; Operation 2 : Out-of-Order Model
        MVI A, 15H
        MVI B, 05H
        MVI C, 03H
        MOV D, B
        MOV E, C
        MOV A, D
        ADD E
        STA 8002H

; Operation 3 : Speculative Execution
        MVI A, 01H
        CPI 01H
        JZ SPECULATE
        MVI B, 00H
        JMP SPEC_END
SPECULATE:
        MVI B, 55H
        MOV A, B
        STA 8003H
SPEC_END:

; Operation 4 : Hyper-Threading Model
        MVI A, 10H
        MVI B, 05H
        ADD B
        STA 8004H
        MVI A, 20H
        MVI B, 03H
        ADD B
        STA 8005H

; Operation 5 : SMT Model
        MVI A, 08H
        MVI B, 02H
        ADD B
        STA 8006H
        MVI A, 06H
        MVI B, 04H
        ADD B
        STA 8007H

; Operation 6 : Throughput Comparison
        MVI A, 04H
        STA 8008H
        MVI A, 02H
        STA 8009H

        HLT

## 2. Description of Operations

### Operation 1 – Superscalar Execution

Independent calculations are performed: 10H + 20H = 30H and 05H + 03H = 08H. The results are stored at 8000H and 8001H. The independent operations model instructions that could be issued in parallel when sufficient execution resources exist.

### Operation 2 – Out-of-Order Execution

Independent operands B = 05H and C = 03H are copied to D and E and used for 05H + 03H = 08H. The result is stored at 8002H. This models executing ready independent work without waiting for unrelated work.

### Operation 3 – Speculative Execution

The program predicts that the conditional branch will be taken. The predicted path loads 55H and stores it at 8003H, modelling execution before the branch outcome is fully confirmed.

### Operation 4 – Hyper-Threading Model

Two independent instruction streams are represented. Thread 1 produces 10H + 05H = 15H and Thread 2 produces 20H + 03H = 23H. Results are stored at 8004H and 8005H.

### Operation 5 – SMT Model

Two independent thread calculations are represented: 08H + 02H = 0AH and 06H + 04H = 0AH. Results are stored at 8006H and 8007H. This models simultaneous use of execution resources by multiple threads.

### Operation 6 – Throughput Comparison

The sequential model is represented by 04H operations, while the parallel model is represented by 02H operations per group. These are stored at 8008H and 8009H to show the conceptual throughput improvement.

Note: Q3–Q5 are conceptual simulations using 8085 instructions. The 8085 does not natively implement a modern five-stage pipeline, superscalar issue, out-of-order execution, speculative execution, hyper-threading, or SMT.